# ROBOSOCCER 4.0

## Control Engineering Challenge

## 1. Challenge

Robosoccer 4.0 is not a speed-testing contest.

It is a control engineering challenge designed to evaluate how well participants can:

- Translate discrete commands into smooth motion

- Control momentum in a physics-based environment

- Maintain stability during ball interaction

- Optimise movement for goal scoring under time pressure

Participants are expected to demonstrate engineering judgment, not brute force.

## 2. Simulation Environment

### 2.1 Arena

- Dimensions: 15 m × 15 m

- Surface: Flat, uniform friction

- Single ball, standard physics

- Goals are located at the opposite end

- Simulation platform: CoppeliaSim Edu

### 2.2 Robot Platform

- Differential drive mobile robot

- Four revolute wheel joints

- No modification permitted to:

  - Robot geometry

  - Mass or inertia

  - Friction coefficients

  - Physics engine settings

3. Control Interface

### 3.1 Command Source

Participants receive commands from an organiser-provided browser controller.

Each command is discrete and represents a user intent, not a motor instruction.

Example:

{ "cmd": "forward" }

### 3.2 Allowed Commands

| Command | Intent |
| --- | --- |
| forward | Advance robot |
| backward | Reverse robot |
| left | Rotate left |
| right | Rotate right |
| stop | Apply braking |

Participants must convert these intents into physically reasonable wheel velocities.

## 4. Motion Constraints & Engineering Rationale

### 4.1 Speed Limits (Mandatory)

Maximum wheel angular velocity: 5.0 rad/s

Turning speed reduction factor: 0.6

These limits are derived from:

- Arena size

- Match duration

- Dynamic stability

- Ball interaction realism

4.2 Acceleration Constraint (Critical)

Instant velocity jumps are not permitted.

Directly setting wheel velocity from 0 → MAX_SPEED in one simulation step results in:

- Wheel slip

- Ball teleportation effects

- Unrealistic motion

- Loss of control

Instead, participants must implement velocity ramping.

## 5. Control Tasks

### Task 1 — Velocity Ramping (Fundamental Control)

Objective:
Ensure that wheel velocities increase and decrease gradually, not instantaneously.

Requirements:

- Velocity must ramp toward the target value

- Sudden direction changes must pass through zero

- Braking must feel controlled, not abrupt

Learning outcomes:

- Understanding of actuator dynamics

- Control smoothness

- Stability awareness

### Task 2 — Controlled Turning Under Motion

Objective:
Turn the robot while preserving forward momentum stability.

Requirements:

- Turning reduces forward speed

- No wheel reversal at high speed

- Turning must not cause spinning in place unless commanded

Learning outcomes:

- Differential drive understanding

- Angular vs linear velocity balance

### Task 3 — Direction Change Discipline

Objective:
Prevent unrealistic motion during rapid command changes.

Scenario Examples:

- forward → backward

- left → right

- forward → left

Requirements:

- Velocity must decay before reversing

- No instantaneous sign change at maximum speed

Learning outcomes:

- Control logic robustness

- Real-world motor behaviour modelling

### Task 4 — Precision Stop & Recovery

Objective:
Stop near the ball or goal without overshooting.

Requirements:

- stop command must decelerate, not freeze

- The robot must settle within a short distance

- Resume motion smoothly afterwards

Learning outcomes:

- Braking logic

- Positioning awareness

## 6. Final Match (Competitive Round)

- Duration: 3 minutes

- Objective: Maximum goals scored

- All motion constraints remain enforced

## 7. Reference Control Structure (Guideline)

Participants are encouraged (not forced) to follow this pattern:

Command → Target Velocity

Target Velocity → Ramped Velocity

Ramped Velocity → Joint Actuation

This structure reflects real robotic motor control pipelines.

## 8. Evaluation Criteria

Judging will consider:

| Criterion | Weight |
| --- | --- |
| Control smoothness | High |
| Turning discipline | High |
| Ball handling | High |
| Responsiveness | Medium |

We wish you clarity in design, confidence in execution, and enjoyment throughout the challenge. Compete fairly, think critically, and most importantly—have fun building something that moves because you made it move well.

We look forward to seeing your ideas come alive on the field.

All the best, and welcome to Robosoccer 4.0.

Prepared by

ROBOSOCCER 4.0

TEAM

In case you find any issues, kindly let us know 23f3100008@es.study.iitm.ac.in 24dp1000036@ds.study.iitm.ac.in